Cần fix:

XÓA!
- Ở phần phép tính khi đưa ra kết quả: tìm cách thể hiện thêm 1 dòng: Đây là kết quả ước tính gì đó, để chi tiết hơn thì hãy để đội kỹ thuật tưvaans chính xác hơn…. kiểu vậy

- Cật nhật công thức:

# BẢN 2 — GỬI CHO CODER

Đây là bản tôi khuyên bạn gửi gần như nguyên văn.

// ==================================================

// VIETHUNGSOLAR - SOLAR SAVINGS CALCULATOR

// Version assumptions: 2026

// ==================================================

const CONFIG = {

// VAT

vatRate: 0.08,

// Solar offset ratio

offsetRatio: {

household: 0.80,

production: 0.70,

business: 0.70,

},

// Solar production assumption

monthlyYieldPerKwp: 120, // kWh/kWp/month

// Estimated investment cost

systemCostPerKwp: 12_000_000, // VND/kWp

// Round system size upward to nearest 0.5 kWp

powerRoundStep: 0.5,

// Default business/factory operating period

operatingHours: {

start: "06:00",

end: "18:00",

// Based on 2026 TOU periods:

normalHours: 11.5, // 06:00 - 17:30

peakHours: 0.5,    // 17:30 - 18:00

lowHours: 0

}

};

## A. Biểu giá hộ gia đình

const HOUSEHOLD_RATES = [

{

limit: 50,

priceBeforeVat: 1984,

priceWithVat: 1984 * 1.08

},

{

limit: 50,

priceBeforeVat: 2050,

priceWithVat: 2050 * 1.08

},

{

limit: 100,

priceBeforeVat: 2380,

priceWithVat: 2380 * 1.08

},

{

limit: 100,

priceBeforeVat: 2998,

priceWithVat: 2998 * 1.08

},

{

limit: 100,

priceBeforeVat: 3350,

priceWithVat: 3350 * 1.08

},

{

limit: Infinity,

priceBeforeVat: 3460,

priceWithVat: 3460 * 1.08

}

];

Tương đương:

2142.72

2214.00

2570.40

3237.84

3618.00

3736.80

### Hàm kWh → tiền điện

function calculateHouseholdBill(kwh) {

let remaining = kwh;

let bill = 0;

for (const tier of HOUSEHOLD_RATES) {

if (remaining <= 0) break;

const used = Math.min(remaining, tier.limit);

bill += used * tier.priceWithVat;

remaining -= used;

}

return bill;

}

### Hàm tiền điện → kWh

function householdBillToKwh(monthlyBill) {

const thresholds = [

{

maxBill: 107136,

baseKwh: 0,

baseBill: 0,

rate: 2142.72

},

{

maxBill: 217836,

baseKwh: 50,

baseBill: 107136,

rate: 2214.00

},

{

maxBill: 474876,

baseKwh: 100,

baseBill: 217836,

rate: 2570.40

},

{

maxBill: 798660,

baseKwh: 200,

baseBill: 474876,

rate: 3237.84

},

{

maxBill: 1160460,

baseKwh: 300,

baseBill: 798660,

rate: 3618.00

}

];

for (const tier of thresholds) {

if (monthlyBill <= tier.maxBill) {

return (

tier.baseKwh +

(monthlyBill - tier.baseBill) / tier.rate

);

}

}

// Above 400 kWh

return (

400 +

(monthlyBill - 1160460) / 3736.80

);

}

# B. Biểu giá nhà xưởng / sản xuất

Giá gốc chưa VAT:

const PRODUCTION_RATES = [

{

voltage: ">=110kV",

normal: 1811,

low: 1146,

peak: 3266

},

{

voltage: "22-110kV",

normal: 1833,

low: 1190,

peak: 3398

},

{

voltage: "6-22kV",

normal: 1899,

low: 1234,

peak: 3508

},

{

voltage: "<6kV",

normal: 1987,

low: 1300,

peak: 3640

}

];

Do không biết điện áp, lấy trung bình:

const PRODUCTION_AVG_WITH_VAT = {

normal: 2033.10,

low: 1314.90,

peak: 3729.24

};

Giá đại diện cho thời gian sử dụng 06:00–18:00:

const PRODUCTION_EFFECTIVE_RATE =

(

PRODUCTION_AVG_WITH_VAT.normal * 11.5 +

PRODUCTION_AVG_WITH_VAT.peak * 0.5 +

PRODUCTION_AVG_WITH_VAT.low * 0

) / 12;

// Result:

// 2103.7725 VND/kWh

### Dùng trong calculator

const PRODUCTION_RATE = 2103.7725;

Không nên lưu 2104 để tính toán; chỉ làm tròn thành 2.104 đ/kWh khi hiển thị.

# C. Biểu giá doanh nghiệp / kinh doanh

const BUSINESS_RATES = [

{

voltage: ">=22kV",

normal: 2887,

low: 1609,

peak: 5025

},

{

voltage: "6-22kV",

normal: 3108,

low: 1829,

peak: 5202

},

{

voltage: "<6kV",

normal: 3152,

low: 1918,

peak: 5422

}

];

Trung bình cấp điện áp + VAT:

const BUSINESS_AVG_WITH_VAT = {

normal: 3292.92,

low: 1928.16,

peak: 5633.64

};

Giá cho thời gian 06:00–18:00:

const BUSINESS_EFFECTIVE_RATE =

(

BUSINESS_AVG_WITH_VAT.normal * 11.5 +

BUSINESS_AVG_WITH_VAT.peak * 0.5 +

BUSINESS_AVG_WITH_VAT.low * 0

) / 12;

// Result:

// 3390.45 VND/kWh

Website dùng:

const BUSINESS_RATE = 3390.45;

# D. Hàm làm tròn công suất

function roundUpPower(value) {

const step = CONFIG.powerRoundStep;

return Math.ceil(value / step) * step;

}

Ví dụ:

7.01 → 7.5

7.50 → 7.5

7.51 → 8.0

8.01 → 8.5

# E. Calculator nhà ở hoàn chỉnh

function calculateHousehold(monthlyBill) {

// 1. Estimate current monthly consumption

const energy = householdBillToKwh(monthlyBill);

// 2. Offset 80%

const offsetEnergy =

energy * CONFIG.offsetRatio.household;

// 3. Remaining grid electricity

const remainingEnergy =

energy * (1 - CONFIG.offsetRatio.household);

// 4. Required solar capacity

const rawPower =

offsetEnergy / CONFIG.monthlyYieldPerKwp;

const recommendedPower =

roundUpPower(rawPower);

// 5. Recalculate electricity bill

// using the progressive residential tariff

const billAfterSolar =

calculateHouseholdBill(remainingEnergy);

// 6. Monthly saving

const monthlySaving =

monthlyBill - billAfterSolar;

// 7. Investment

const investment =

recommendedPower *

CONFIG.systemCostPerKwp;

// 8. Payback

const annualSaving =

monthlySaving * 12;

const paybackYears =

investment / annualSaving;

return {

energy,

offsetEnergy,

remainingEnergy,

rawPower,

recommendedPower,

currentBill: monthlyBill,

billAfterSolar,

monthlySaving,

annualSaving,

investment,

paybackYears

};

}

# F. Calculator nhà xưởng

function calculateProduction(monthlyBill) {

const rate = PRODUCTION_RATE;

// Current kWh

const energy =

monthlyBill / rate;

// Solar offsets 70%

const offsetEnergy =

energy * CONFIG.offsetRatio.production;

const remainingEnergy =

energy * 0.30;

// Solar capacity

const rawPower =

offsetEnergy /

CONFIG.monthlyYieldPerKwp;

const recommendedPower =

roundUpPower(rawPower);

// Bill after solar

const billAfterSolar =

remainingEnergy * rate;

const monthlySaving =

monthlyBill - billAfterSolar;

const annualSaving =

monthlySaving * 12;

const investment =

recommendedPower *

CONFIG.systemCostPerKwp;

const paybackYears =

investment / annualSaving;

return {

energy,

offsetEnergy,

remainingEnergy,

rawPower,

recommendedPower,

effectiveElectricityRate: rate,

currentBill: monthlyBill,

billAfterSolar,

monthlySaving,

annualSaving,

investment,

paybackYears

};

}

# G. Calculator doanh nghiệp

function calculateBusiness(monthlyBill) {

const rate = BUSINESS_RATE;

const energy =

monthlyBill / rate;

const offsetEnergy =

energy * CONFIG.offsetRatio.business;

const remainingEnergy =

energy * 0.30;

const rawPower =

offsetEnergy /

CONFIG.monthlyYieldPerKwp;

const recommendedPower =

roundUpPower(rawPower);

const billAfterSolar =

remainingEnergy * rate;

const monthlySaving =

monthlyBill - billAfterSolar;

const annualSaving =

monthlySaving * 12;

const investment =

recommendedPower *

CONFIG.systemCostPerKwp;

const paybackYears =

investment / annualSaving;

return {

energy,

offsetEnergy,

remainingEnergy,

rawPower,

recommendedPower,

effectiveElectricityRate: rate,

currentBill: monthlyBill,

billAfterSolar,

monthlySaving,

annualSaving,

investment,

paybackYears

};

}

# H. Một hàm chung cho toàn website

Coder có thể gọi:

function calculateSolar({

customerType,

monthlyBill

}) {

switch (customerType) {

case "household":

return calculateHousehold(monthlyBill);

case "production":

return calculateProduction(monthlyBill);

case "business":

return calculateBusiness(monthlyBill);

default:

throw new Error("Invalid customer type");

}

}

Map giao diện của bạn thành:

const CUSTOMER_TYPE_MAP = {

"Nhà ở": "household",

"Nhà xưởng": "production",

"Nhà máy": "production",

"Công nghiệp": "production",

"Trang trại": "production",

"Văn phòng": "business",

"Doanh nghiệp": "business",

"Kinh doanh": "business"

};

# KẾT QUẢ WEBSITE NÊN TRẢ VỀ

Tôi khuyên calculator nội bộ tính nhiều biến, nhưng phía khách hàng chỉ cần thấy 3 kết quả chính:

Công suất hệ thống đề xuất: 8.0 kWp

Tiết kiệm mỗi tháng: ≈ 3,4 triệu

Thời gian hoàn vốn: ≈ 2,3 năm

Có thể thêm một dòng nhỏ:

Kết quả mang tính ước tính, dựa trên mức sử dụng điện, biểu giá điện hiện hành và giả định sản lượng điện mặt trời trung bình.